| Project Number: 14011 | Document Type: Software Requirement Specification |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 18-Apr-2023 | Product Name: Template |  |  | Revision: A |
| History Rev; Date; Comment; Author. A; 18-Apr-2023; Initial; Thomas Wiedebusch. |
|  |  | Copyright © 2021..2023 by SENSUS GmbH CONFIDENTIAL |  |  |
| File: Template SRS .docx |  |  |  |  |

Reviews

| Rev | Date | Reviewer | Department |
| --- | --- | --- | --- |

List Of Contents

1.	Introduction	6

1.1	Content	6

1.2	System Overview	6

1.3	FW-Update Software Functions	7

2.	Prerequisites	8

2.1	Hardware Requirements	8

2.2	Software Requirements	8

3.	Installation	8

3.1	FW-Update Software Installation	8

3.2	Driver Installation and Setup	8

4.	Requirements	9

4.1	Calculations	9

4.1.1	Calculation of DUT-Volume based on Meter Measurements	9

4.1.2	Calculation of DUT-Time based on Meter Measurements	9

4.1.3	Calculation Of DUT-Flow-Rate	9

4.1.4	Calculation Of DUT-Volume Based On Flow-Rate	10

4.1.5	Calculation Of Calibration-Factor	10

4.1.6	Calculation of Deviation	10

4.2	Error Handling	11

4.2.1	Channel Assignment	11

5.	Trouble Shooting	12

6.	Contacts	12

7.	References	12

List Of Tables

Table 1.1 Abbreviations	5

Table 9.1 Contacts	12

List Of Figures

Figure 1.1 FW-Update with USB-IrDA – Offline Operation	6

List of Equations

Equation 2.1 DUT-Volume based on Meter Measurements	9

Equation 2.2 DUT-Time based on Meter Measurements	9

Equation 2.3 Calculation of DUT-Flow-Rate	9

Equation 2.4 Calculation of DUT-Volume based on Flow-Rate	10

Equation 2.5 Calculation of Calibration-Factor	10

Equation 2.6 Calculation of Deviation relative	10

Equation 2.7 Calculation of Deviation in Percent	10

Abbreviations

| Builder Operator | Operator of the “FW-Update Builder“ assembling the “FW-Update Safe” |
| --- | --- |
| DLL | Dynamic Link Library |
| FW | Firmware, hardware depending software |
| FW-Update | Firmware update |
| FW-Update Builder | Builder of the “FW-Update Safe”, tool to select all needed components and to encrypt the “FW-Update Safe” |
| FW-Update Loader | Software for decryption of the “FW-Update Safe” and host for the “FW-Update Software” |
| FW-Update Process | The process of installing the firmware |
| FW-Update Safe | Encrypted container of all components needed for the “FW-Update Process” during offline operation |
| FW-Update SW | Software executing the “FW-Update Process” |
| Genesis | Legacy Name of Cordonel |
| HW | Hardware |
| Offline Operation | Without connection to the Xylem-Network |
| Online Operation | Directly or via VPN connected to the Xylem-Network |
| PCB ID | Printed Circuit Bord Identifier, unique identification number of the Cordonel HW |
| SRS | Software Requirement Specification |
| SW | Software |
| Update Opertor | Operator executing the “FW-Update Process“ |
| VPN | Virtual Private Network |

Table 1.1 Abbreviations

# Introduction

This document is a template for the “Software Requirement Specification”.

## Content

The document covers the requirements for the XXX-product.

## System Overview

Two hardware systems can be used for the firmware update:

- USB-IrDA: The IrDA Plug-In adaptor is directly connected via USB to the PC. No driver installation is required.

- Micro-MOXA: The IrDA Plug-In adaptor is connected via a box containing an interface with a high noise immunity for harsh environments. A driver needs to be installed to support the RS485-4W interface.

Figure 1.1 FW-Update with USB-IrDA – Offline Operation

## FW-Update Software Functions

The “FW-Update SW” is the worker who updates the Cordonel, logs all processing steps, being able to analyse those on later inspection, and restores some important settings. Due to the strong update capability check, it avoids an installation of incompatible firmware.

The “FW-Update SW” base functions:

- Auto-detection of communication port and connected Cordonel,

- Generating a “FW-Update Report” file with all executed processing steps,

- Switching OFF the pulse output on IrDA to free the IrDA for standard communication and reminding the original settings of the pulse output to a file,

- Authentication with the password level 8 or, on missing or destroyed password file, with the skeleton key,

- Activate the radio using the customer encryption key,

- Store all previously adjusted settings,

- Reading out the installed firmware of the Cordonel before the update,

- Carrying out an update-capability-check for “Core Revision”, “Metrology Version”, “Region”, “Radio Frequency” and “Meter Size”,

- Lock update on failed update-capability-check,

- Reading out all configurations before the update,

- Listing all installed files on the Cordonel before the update,

- Free memory space for the update procedure by erasing unnecessary files from the Cordonel file system,

- Carrying out the FW-Update procedure,

- Reading out all configurations after the update,

- Listing all files after the update,

- Restoring necessary files for operational need which previously may have been erased,

- Restoring the password file,

- Validate password file by login with password level 8,

- Release display to normal operation for accumulated volume and flow rate,

- Restoring the pulse output to original settings.

# Prerequisites

## Hardware Requirements

- PC with MS-Windows™ 10 Operating system.

- USB-IrDA or Micro-MOXA with IrDA head.

## Software Requirements

Microsoft™ .NET framework 4.7.2 has to be installed in advance.

# Installation

## FW-Update Software Installation

.

## Driver Installation and Setup

# Requirements

This chapter specifies the requirements of the product.

## Calculations

### Calculation of DUT-Volume based on Meter Measurements

[SRS_TBF_COMMON_METRO_001]

For the “Static Start/Stop” the measurement-time is out of focus because the reference volume will be obtained from a weight-scale or a reference meter. Therefore the DUT-volume can be directly calculated using the difference between the start- and stop-volume taking an overflow into account:

VDUTm3= (VDUTstopm3+VDUT overflow[m3])-VDUTstart[m3]

Equation 4.1 DUT-Volume based on Meter Measurements

Error handling:

A status shall be set, if the volume of the DUT is zero.

### Calculation of DUT-Time based on Meter Measurements

[SRS_TBF_COMMON_METRO _002]

For the DUT-time can be directly calculated using the difference between the start- and stop-time given by the meter:

tDUTs= (tDUTstops+ tDUToverflow[s])-tDUTstart[s]

Equation 4.2 DUT-Time based on Meter Measurements

Error handling:

A status for timings below and equal to zero shall be set.

### Calculation Of DUT-Flow-Rate

[SRS_TBF_COMMON_METRO _003]

For the flow-rate calculation the individual DUT-volume and the DUT-time is being needed:

QDUTm³h= VDUT[m3]tDUT[s]∙3600sh

Equation 4.3 Calculation of DUT-Flow-Rate

### Calculation Of DUT-Volume Based On Flow-Rate

[SRS_TBF_COMMON_METRO _004]

For the “Flying Start/ Stop Measurement” the reference time is being used to recalculate the volume during the test time, due to the fact that the start and stop data record may be delayed by the software program flow or the unpredictable operational system timing:

VDUT[m3]= QDUTm3h ∙ tREFs∙h3600s

Equation 4.4 Calculation of DUT-Volume based on Flow-Rate

### Calculation Of Calibration-Factor

[SRS_TBF_COMMON_METRO _005]

The calibration is being done using the measured volume of each individual path, the reference volume, measured by a weight scale or a REF, and for the flying start/stop measurement the timings of the individual channel and the precisely timing of the reference system.

These values will be used to calculate the new calibration factor sx. It is based on the default scale factor  sdefx multiplied by the desired deviation k at this specific flow-rate. The desired deviation is needed to adjust the curve of deviations depending on different flow rates into an envelope of tolerances. Detailed information of the calibration procedure for the Genesis-water-meter can be found in [Fehler! Verweisquelle konnte nicht gefunden werden.]. The scale-factor shall be based on absolute values to be independent of the direction of the flow:

sx = |VREF|VDUTx  ∙ sdefx ∙  k          x=1..3;  0.95<k<1.05

Equation 4.5 Calculation of Calibration-Factor

Error handling:

A scale factor of zero shall be avoided with the status of REF-volume equals to 0.0!

### Calculation of Deviation

[SRS_TBF_COMMON_METRO _006]

The deviation dQ at a specific flow rate Q shall be calculated using the reference and the measured volume. The deviation calculation shall be based on absolute values to be independent of the direction of the flow rate:

dQrel[ ]= |VDUT|-|VREF||VREF|

Equation 4.6 Calculation of Deviation relative

dQ%=  dQrel ∙100%

Equation 4.7 Calculation of Deviation in Percent

## Error Handling

Some errors may avoid the ability to calculate the metrology values.

### Channel Assignment

[SRS_TBF_COMMON_METRO _007]

It shall be checked if the required and the start- and end-record-channels are identical to avoid comparison of unequally channels.

# Trouble Shooting

…

# Contacts

| Name | Funktion | Contact |
| --- | --- | --- |
| Thomas Wiedebusch | Software development | thomas.wiedebusch@t-wiedebusch.de +49 179 146 04 93 |
| Jens Schulz | Productline - Manager | jens.schulz@xylem.com +49 5102 743 195 |
| Roland Drabesch | Lead - Software development | roland.drabesch@xylem.com +49 5102 743 225 |

Table 6.1 Contacts

# References

[FWU_MRD]	Cordonel Service FW-Update Process MRD; Published:  SENSUS GmbH; Author: Thomas Wiedebusch; Edition: September 2020 - Revision B

[FWU_BLD]	Quick Reference Cordonel Service FW-Update Builder; Published:  SENSUS GmbH; Author: Thomas Wiedebusch; Edition: Mar 2023 - Revision D

[FWU_LDR]	Quick Reference Cordonel Service FW-Update Loader; Published:  SENSUS GmbH; Author: Thomas Wiedebusch; Edition: April 2021 - Revision A

[FWU_PK]	Quick Reference Cordonel Service FW-Update Packet; Published:  SENSUS GmbH; Author: Thomas Wiedebusch; Edition: April 2021 - Revision A